Product Specification

| Project Name: | Carter-Hoffmann PUC 10.1" |
| --- | --- |
| Model No.: | 90LM101C109A1-CH |
| Description: | Promate Midband 10.1" HLUI-Lite HMI |
| Document No.: | 2400002421 |
| Version: | V01 |
| Total Pages: | 22 |
| Date: | 2026/10/01 |

| Approved By | Checked By | Prepared By |
| --- | --- | --- |
| Howard Kuo | Marius Reyes | Charlie Wang |

Record of Revision

| Version | Revise Date | Page Affected | Content |
| --- | --- | --- | --- |
| 01 | 2026/10/01 | - | First release, compiled from department design specifications |

Contents

1. General Description	4

2. Product Specification	5

2.1 System Overview	5

2.2 Display & Optical Characteristics	5

2.2.1 Display Characteristics	5

2.2.2 Optical Characteristics	5

2.3 Embedded Board Specification	6

2.3.1 Hardware Features	6

2.3.2 Pin Assignment	6

2.3.3 Function Block	16

2.3.4 Connector & PCB Schematic	17

2.4 CTP Touch Panel Specification	18

2.5 Touch Panel Defects & Acceptance	19

2.6 Mechanical Outline Drawing	20

2.7 Product Explode Drawing	21

3. Package (Air)	22

# 1. General Description

The module is a 10.1-inch commercial food-service midband touch HMI display, integrating a TFT LCD module optically bonded to a capacitive touch panel, an embedded single-board computer, and a mechanical chassis assembly into a single ready-to-mount unit.

Main components:

- 10.1" TFT LCD Module (LCM), 1920 × 1200 resolution, optically bonded to the touch panel

- 10.1" Projected Capacitive Touch Panel (PCT), glass-glass structure, I2C interface

- Embedded system board (NXP i.MX8M Mini processor), supporting LCD/touch, Ethernet, RS232/RS485, CAN bus, USB and audio

- Mechanical chassis, midband, EMI cover

Figure 1 – Product Appearance – Front View

Figure 2 – Product Appearance – Rear View

# 2. Product Specification

## 2.1 System Overview

Table 1 – System Key Features Overview

| Category | Key Features |
| --- | --- |
| Display Solution | TDM, 10.1", 1920 × 1200 |
| Touch Solution | Microchip maXTouch Series |
| Processor / MCU | NXP i.MX8M Mini |
| Memory | eMMC Flash 16 GB, LPDDR4 DRAM 2 GB, Micro SD Card (4-bit SDHC/SDIO) |
| I/Os | 1 × 10/100 Ethernet, RS485 × 2, RS232 × 1, CAN Bus × 1, USB (OTG 2.0 × 1, Host 2.0 × 4), GPIO/I2C × 6, PWM × 2 |
| Power Supply | 24 VDC ± 10% |
| OS / Firmware | Android 9 |
| Application | Blue Sparq + Vantron BlueSphere |
| Operating Temp. & Humidity | 0°C ~ 70°C; 20 ~ 90% RH (Non-condensing) |
| Storage Temp. & Humidity | -20°C ~ 70°C; 20 ~ 90% RH (Non-condensing) |
| Product Dimension | 258.50 (W) × 171.50 (H) × 30.80 (D) mm |

## 2.2 Display & Optical Characteristics

### 2.2.1 Display Characteristics

Table 2 – Display Characteristics

| Item | Unit | Specification | Remark |
| --- | --- | --- | --- |
| Screen Diagonal | inch | 10.1 |  |
| Outline | mm | 254.9 (W) × 167.9 (H) |  |
| Active Area | mm | 216.81 (H) × 135.5 (V) |  |
| Pixels (H × V) | - | 1920 × 1200 |  |
| Pixel Pitch | mm | 0.11292 × 0.11292 |  |
| Pixel Arrangement | - | RGB vertical stripe |  |
| Display Mode | - | Normally black |  |
| Support Color | - | 16.7M |  |
| Cover Glass Hardness | - | 6H |  |

### 2.2.2 Optical Characteristics

Table 3 – Optical Characteristics

| Item | Unit | Min. | Typ. | Max. |
| --- | --- | --- | --- | --- |
| Viewing Angle – Horizontal (Right/Left), CR≥10 | ° | 80 / 80 | 85 / 85 | - |
| Viewing Angle – Vertical (Up/Down), CR≥10 | ° | 80 / 80 | 85 / 85 | - |
| Contrast Ratio (Normal Direction) | - | 800 | 1000 | - |
| White Color Coordinate x | - | 0.27 | 0.32 | 0.37 |
| White Color Coordinate y | - | 0.29 | 0.34 | 0.39 |
| Luminance Uniformity (9 Points) | % | 75 | 80 | - |
| White Luminance | cd/m² | 510 | 630 | - |

## 2.3 Embedded Board Specification

### 2.3.1 Hardware Features

Table 4 – Embedded Board Hardware Features

| Category | Item | Specification |
| --- | --- | --- |
| Processor |  | NXP i.MX8M Mini |
| Memory | eMMC Flash | 16 GB |
|  | EEPROM | 4 Kbit |
|  | LPDDR4 DRAM | 2 GB |
|  | Micro SD Card | 4-bit SDHC/SDIO |
| Communication Interface | Network | 1 × 10/100 Ethernet |
|  | RS485 Port | × 2 |
|  | RS232 Port | × 1 |
|  | Console Port | × 1 (Housing) |
|  | CAN Bus | × 1 |
|  | GPIO / I2C | I2C / GPIO × 6, PWM × 2 |
|  | USB Port | USB Type-B OTG 2.0 High Speed × 1; Host 2.0 High Speed × 4 |
|  | Tact Switch | Reset × 1, Power ON/OFF × 1, Bootload × 1 |
| Display | LVDS Connector | 1×40 Pin, P=0.5 mm, 90° SMD |
|  | Touch Connector | I2C interface / USB |
|  | Backlight Connector | 1×7 Pin, P=2.0 mm, 180° DIP (5 V/12 V) |
| Audio | Speaker Output | 1 × 2-Pin connector, 10 W / 8 Ω |
| Board Dimension | Size | 155 mm × 90 mm |
| Power | Input Power Voltage | 24 VDC ± 10% |
|  | RTC | 3 V lithium battery, Type CR1220 |

### 2.3.2 Pin Assignment

The embedded board provides the following connectors. The connector list and pin definitions are summarized below.

Figure 3 – Connector Location Diagram

Table 5 – Connector List

| No. | Ref. | Function | Equivalent Connector |
| --- | --- | --- | --- |
| 1 | J1 | Power In | B3B-XH-A(LF)(SN) |
| 2 | J2 | LVDS | I-PEX-0553-SMTR-40P |
| 3 | J4 | Speaker-R | B2B-PH-K-S(LF)(SN) JST |
| 4 | J4-1 | Speaker-L | B2B-PH-K-S(LF)(SN) JST |
| 5 | J5 | USB-A | 614004135023 |
| 6 | J6 | USB-A | 614004135023 |
| 7 | J7 | RS485 1/2 | 705430108 |
| 8 | J8 | USB (Housing) | B4B-PH-K-S(LF)(SN) JST |
| 9 | J9 | USB (Housing) | B4B-PH-K-S(LF)(SN) JST |
| 10 | J10 | USB-B | 61400413321 |
| 11 | J11 | Ethernet | KRV-01F-1-D12-180(01) |
| 12 | J12 | CAN Bus | 705430002 |
| 13 | J14 | I2C | 530470410 |
| 14 | J16 | GPIO / I2C | 3020-14-0300-00 CNC |
| 15 | J17 | QSPI | 640456-8 |
| 16 | J20 | RS232 / RS485 | 449140801 |
| 17 | J21 | Buzzer | B2B-PH-K-S(LF)(SN) JST |
| 18 | J23 | MIC | 53398-0267 |
| 19 | J24 | PB | 53398-0367 |
| 20 | J25 | Touch | 53398-0667 |
| 21 | J26 | RS232 (Debug) | TU5005WNV-1X06-NL-1 |
| 22 | J27 | Test-5V | 53398-0267 |
| 23 | CON1 | RS232 (Debug) | TU1252-WNR-05S |
| 24 | CON2 | Backlight | B7B-PH-K-S(LF)(SN) JST |

J1 : Power In - B3B-XH-A(LF)(SN)

Table 6 – J1 : Power In - B3B-XH-A(LF)(SN) Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | System Power Input | Main power input: 24 VDC ± 10% |
| 3 | GND | Ground |

J2 : LVDS - I-PEX-0553-SMTR-40P

Table 7 – J2 : LVDS - I-PEX-0553-SMTR-40P Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | LCD_PWR | (3V3) 5 V default |
| 3 | LCD_PWR | (3V3) 5 V default |
| 4 | LCD_PWR | (3V3) 5 V default |
| 5 | GND | Ground |
| 6 | GND | Ground |
| 7 | LVDS0_TX0_N | LVDS0 data |
| 8 | LVDS0_TX0_P | LVDS0 data |
| 9 | LVDS0_TX1_N | LVDS0 data |
| 10 | LVDS0_TX1_P | LVDS0 data |
| 11 | LVDS0_TX2_N | LVDS0 data |
| 12 | LVDS0_TX2_P | LVDS0 data |
| 13 | GND | Ground |
| 14 | LVDS0_CLK_N | LVDS0 clock |
| 15 | LVDS0_CLK_P | LVDS0 clock |
| 16 | GND | Ground |
| 17 | LVDS0_TX3_N | LVDS0 data |
| 18 | LVDS0_TX3_P | LVDS0 data |
| 19 | U/D | Up/Down |
| 20 | LVDS1_TX0_N | LVDS1 data |
| 21 | LVDS1_TX0_P | LVDS1 data |
| 22 | LVDS1_TX1_N | LVDS1 data |
| 23 | LVDS1_TX1_P | LVDS1 data |
| 24 | LVDS1_TX2_N | LVDS1 data |
| 25 | LVDS1_TX2_P | LVDS1 data |
| 26 | GND | Ground |
| 27 | LVDS1_CLK_N | LVDS1 clock |
| 28 | LVDS1_CLK_P | LVDS1 clock |
| 29 | GND | Ground |
| 30 | LVDS1_TX3_N | LVDS1 data |
| 31 | LVDS1_TX3_P | LVDS1 data |
| 32 | SELB | Select 6/8 bit |
| 33 | VLED_PWM | LED PWM |
| 34 | VLED_EN | LED enable |
| 35 | STBYB | Standby |
| 36 | L/R | Left/Right |
| 37 | RESET | Reset |
| 38 | Backlight | (12 V) 5 V default |
| 39 | Backlight | (12 V) 5 V default |
| 40 | Backlight | (12 V) 5 V default |

J4 (R) / J4-1 (L) : Speaker - B2B-PH-K-S(LF)(SN) JST

Table 8 – J4 (R) / J4-1 (L) : Speaker - B2B-PH-K-S(LF)(SN) JST Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | Audio_R+ / Audio_L+ | Positive PWM output |
| 2 | Audio_R- / Audio_L- | Negative PWM output |

J5 / J6 : USB-A - 614004135023

Table 9 – J5 / J6 : USB-A - 614004135023 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | 5 V | Supply 5 V |
| 2 | USB D- | Data minus |
| 3 | USB D+ | Data plus |
| 4 | GND | Ground |

Note:  The two USB 2.0 headers (J5 / J6) on the board can deliver a combined power rating of 3.5 A.

J7 : RS485 1/2 - 705430108

Table 10 – J7 : RS485 1/2 - 705430108 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | RS485-A | RS485 termination high side |
| 3 | RS485-B | RS485 termination low side |
| 4 | Termination | Termination |

J8 / J9 : USB (Housing) - B4B-PH-K-S(LF)(SN) JST

Table 11 – J8 / J9 : USB (Housing) - B4B-PH-K-S(LF)(SN) JST Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | 5 V | Supply 5 V |
| 2 | USB D- | Data minus |
| 3 | USB D+ | Data plus |
| 4 | GND | Ground |

Note:  The two USB 2.0 headers (J8 / J9) on the board can deliver a combined power rating of 1.5 A.

J10 : USB-B - 61400413321

Table 12 – J10 : USB-B - 61400413321 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | 5 V | Supply 5 V |
| 2 | USB D- | Data minus |
| 3 | USB D+ | Data plus |
| 4 | GND | Ground |

J11 : Ethernet - KRV-01F-1-D12-180(01)

Table 13 – J11 : Ethernet - KRV-01F-1-D12-180(01) Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | TD+ | Media-dependent interface |
| 2 | TCT | VDDIO |
| 3 | TD- | Media-dependent interface |
| 4 | RD+ | Media-dependent interface |
| 5 | RCT | VDDIO |
| 6 | RD- | Media-dependent interface |
| 7 | NC | NC |
| 8 | GND | Ground |
| 9 | D1 | Yellow LED ACT |
| 10 | D2 | Yellow LED Link |
| 11 | D3 | Green LED ACT |
| 12 | D4 | Green LED Link |
| 13 | EGND1 | Chassis ground |
| 14 | EGND2 | Chassis ground |

J12 : CAN Bus - 705430002

Table 14 – J12 : CAN Bus - 705430002 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | CANH | High level CAN bus line |
| 2 | CANL | Low level CAN bus line |
| 3 | GND | Ground |

J14 : I2C - 530470410

Table 15 – J14 : I2C - 530470410 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | VCC | VDD_3V3 |
| 2 | I2C4_SCLB | I2C clock (3.3 V) |
| 3 | I2C4_SDAB | I2C data (3.3 V) |
| 4 | GND | Ground |

J16 : GPIO / I2C - 3020-14-0300-00 CNC

Table 16 – J16 : GPIO / I2C - 3020-14-0300-00 CNC Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | OUTPUT2 | PWM interface |
| 3 | UGPIO1 | DO interface |
| 4 | UGPIO5 | DO interface |
| 5 | UGPIO2 | DO interface |
| 6 | GND | Ground |
| 7 | UGPIO3 | DO interface |
| 8 | I2C2_SDAB | I2C data (3.3 V) |
| 9 | VDD_3V3 | Supply 3V3 |
| 10 | I2C2_SCLB | I2C clock (3.3 V) |
| 11 | UGPIO4 | DO interface |
| 12 | OUTPUT1 | PWM interface |
| 13 | UGPIO6 | DO interface |
| 14 | I2C_5V | Supply 5 V |

J17 : QSPI - 640456-8

Table 17 – J17 : QSPI - 640456-8 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | VDD_3V3 | Supply 3V3 |
| 2 | QSPIA_SCLK | Serial clock |
| 3 | QSPIA_nSS0 | Chip select |
| 4 | GND | Ground |
| 5 | QSPIA_DATA0 | Data line 0 |
| 6 | QSPIA_DATA1 | Data line 1 |
| 7 | QSPIA_DATA2 | Data line 2 |
| 8 | QSPIA_DATA3 | Data line 3 |

J20 : RS232 / RS485 - 449140801

Table 18 – J20 : RS232 / RS485 - 449140801 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | 1RS422_A | RS422 signal A (+) |
| 2 | 1RS422_B | RS422 signal B (-) |
| 3 | 1RS422_Y | RS422 signal Y (+) |
| 4 | 1RS422_Z | RS422 signal Z (-) |
| 5 | 2RS232_RXD | RS232 receiver (RXD) |
| 6 | GND | Ground |
| 7 | 2RS232_TXD | RS232 transmitter (TXD) |
| 8 | GND | Ground |

J21 : Buzzer - B2B-PH-K-S(LF)(SN) JST

Table 19 – J21 : Buzzer - B2B-PH-K-S(LF)(SN) JST Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | OUT+ | Amplifier non-inverting output |
| 2 | OUT- | Amplifier inverting output |

J23 : MIC - 53398-0267

Table 20 – J23 : MIC - 53398-0267 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | MIC | Microphone input |

J24 : PB - 53398-0367

Table 21 – J24 : PB - 53398-0367 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | PB | Push button (high active) |
| 2 | GND | Ground |
| 3 | SR | Smart reset |

J25 : Touch - 53398-0667

Table 22 – J25 : Touch - 53398-0667 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | T_3V3 | Touch power |
| 3 | T_I2C_SCL | I2C clock (3.3 V) |
| 4 | T_I2C_SDA | I2C data (3.3 V) |
| 5 | T_RESET | Reset |
| 6 | T_INT | Interrupt pin |

J26 : RS232 (Debug) - TU5005WNV-1X06-NL-1

Table 23 – J26 : RS232 (Debug) - TU5005WNV-1X06-NL-1 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | NC | NC |
| 3 | NC | NC |
| 4 | 1RS232_TXD | RS232 transmitter (TXD) |
| 5 | 1RS232_RXD | RS232 receiver (RXD) |
| 6 | NC | NC |

J27 : Test-5V - 53398-0267

Table 24 – J27 : Test-5V - 53398-0267 Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | DCDC_5V | DCDC 5 V (for test) |
| 2 | GND | Ground |

CON1 : RS232 (Debug) - TU1252-WNR-05S

Table 25 – CON1 : RS232 (Debug) - TU1252-WNR-05S Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | 1RS232_TXD | RS232 transmitter (TXD) |
| 2 | 1RS232_RXD | RS232 receiver (RXD) |
| 3 | GND | Ground |
| 4 | NC | NC |
| 5 | NC | NC |

CON2 : Backlight - B7B-PH-K-S(LF)(SN) JST

Table 26 – CON2 : Backlight - B7B-PH-K-S(LF)(SN) JST Pin Assignment

| Pin No. | Pin Name | Description |
| --- | --- | --- |
| 1 | Backlight | Supply 5 V(12 V) for backlight |
| 2 | Backlight | Supply 5 V(12 V) for backlight |
| 3 | GND | Ground |
| 4 | GND | Ground |
| 5 | VLED_EN | Backlight enable signal |
| 6 | VLED_PWM | Backlight PWM dimming |
| 7 | GND | Ground |

### 2.3.3 Function Block

Figure 4 – Embedded Board Function Block Diagram

### 2.3.4 Connector & PCB Schematic

PCB dimension: 155 mm × 90 mm.

Figure 5 – PCB Top Placement

Figure 6 – PCB Bottom Placement

## 2.4 CTP Touch Panel Specification

Table 27 – Touch Panel General Information

| Item | Contents |
| --- | --- |
| Sensor Structure | Glass-Glass |
| Surface Hardness | 6H (min.) [by JIS K5400] |
| Ball-Falling Test | 2J (IK07) |
| Resolution (H×V) | 1920 × 1200 Dots |
| Screen Size | 10.1 inches Diagonal |
| Interface Type | I2C |
| Driver IC | Microchip maXTouch Series |
| Connector (Wafer Part) | MX1.25-6P-CB |
| Connector (Mating Housing Part) | HR-A1254H-06P or TU1212HNO-06 |

Table 28 – Touch Panel Electrical Characteristics

| Item | Symbol | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Power Supply Input Voltage | TP_VDD | 3.0 | 3.3 | 3.47 | V |
| I/O Signal Voltage | Signal | - | 3.3 | - | V |

Table 29 – CTP Interface Description

| Interface No. | Name |
| --- | --- |
| 1 | TP_VDD |
| 2 | TP_SCL |
| 3 | TP_SDA |
| 4 | TP_INT |
| 5 | TP_RST |
| 6 | TP_GND |

Power-Up / Reset Requirements: the device must be held in RESET (active low) while the digital and analog power supplies (Vdd, VddIO, AVdd) are powering up. The supplies must reach their nominal values before the RESET signal is de-asserted (goes high).

Figure 7 – Power-Up / Reset Timing Diagram

## 2.5 Touch Panel Defects & Acceptance

Visual inspection is conducted under a light source of 500 ± 100 lux, positioned above and between the glass and the inspector. The inspector observes the glass at a viewing distance of 30 ± 5 cm, with an inspection time of 10 to 15 seconds per piece, within a viewing angle of ±45° with respect to the normal direction of the glass surface.

Table 30 – Defects and Acceptance Standards

| Defect Item | Inspection Criteria |
| --- | --- |
| Dot / Bubble / BM Dot | D≤0.15 mm: unlimited; 0.15 mm<D≤0.4 mm: accept 3 pcs (interval of 10 mm); D>0.4 mm: reject |
| Line Particle | W≤0.05 mm: unlimited; 0.05 mm<W≤0.1 mm, L≤10 mm: accept 6 pcs (interval >10 mm); W>0.1 mm: reject |
| Lens Breakage | X≤0.4 mm, Y≤0.4 mm: accept 1 pc/side; X>0.4 mm, Y>0.4 mm: reject |
| Printing Sawtooth | W≤0.15 mm: accept; W>0.15 mm: reject |
| Cover Glass Crack | Crack extending toward the outer edge: accept; extending inward (toward the view area): reject |
| BM Pin Hole | D≤0.2 mm: accept 2 pcs; D>0.2 mm: reject |

## 2.6 Mechanical Outline Drawing

Figure 8 – Product Outline Drawing

## 2.7 Product Explode Drawing

Figure 9 – Product Explode Drawing

Recommended torque: M3 screws, 4 ± 0.3 kgf-cm; M4 screws, 6 ± 0.5 kgf-cm.

# 3. Package (Air)

Carton size: 386 × 291 × 360 mm. Max capacity: 5 modules/carton. Max weight: 12.1 kg/carton ± 5%. Pallet: Promate standard pallet, 1130 × 1130 × 120 mm (pallet weight 7.25 kg ± 1 kg). 24 cartons/pallet = 120 pcs.

Figure 10 – Package (Air)